MAS 2.0 — Cost & Spec Correction Pack

Source: interview with Mark Somoza (co-founder, Helices Biological Photolithography), 28 Jul 2026, corroborated by helicesbio.com and the ChemRxiv component list (DOI 10.26434/chemrxiv-2024-j4c90). Apply the text in RED. Framing note: for a like-for-like TRI capital comparison, use the INSTRUMENT figure (~€150–170K = optics ~€150K + nucleic-acid synthesizer ~€20K); the ~€200–300K is the fully-loaded end-to-end cost incl. gases, pressure regulators, and a climate-controlled lab.

Chapter 1

§2.2 MAS 2.0 — table, Capital cost row

Currently:  Capital cost | ~$30,000 | tens of $K | MED (est; not itemised in paper text — supplementary component list only)

Change to:  Capital cost | ~€150–170K instrument (optics ~€150K + nucleic-acid synthesizer ~€20K); ~€200–300K fully loaded (incl. gases, regulators, climate-controlled lab) | €150K–300K | MED–HIGH — now SOURCED (developer interview, M. Somoza / Helices, Jul 2026; corroborated by the ChemRxiv component list)

§2.2 MAS 2.0 — table, add spec rows (new)

Change to:  Throughput | up to 786,432 (XGA DMD) or 2,073,600 (1080p DMD) unique sequences | — | HIGH | Helices MAS 2.0 vendor spec

Change to:  Cycle / optics | ~15 s coupling; photodeprotection 60 s (NPPOC) / 30 s (Bz-NPPOC) / 6 s (SPh-NPPOC); 365 nm UV LED source; Texas Instruments DMD | — | HIGH | Helices MAS 2.0 vendor spec

§2.2 MAS 2.0 — prose (after the 'Barriers are optics alignment…' sentence), add:

Change to:  The developer confirms no one has yet independently reproduced the open build: the first in-house build took months (now down to a few weeks), gated chiefly by optical alignment, and the parts are expensive — optics alone ~€150,000, ~€200,000–300,000 end-to-end (M. Somoza, Helices Biological Photolithography, interview Jul 2026). The photolabile amidites (NPPOC / Bz-NPPOC / SPh-NPPOC) are available from several suppliers in three generations.

§5.1 Present-day accessible status — MAS 2.0 row

Currently:  MAS 2.0 | Accessible (DIY, open build) | TRL 5, photolithographic

Change to:  MAS 2.0 | Accessible only to well-resourced labs (open build, but ~€200–300K and optics-alignment-gated; no independent build yet) | TRL 5, photolithographic (developer interview, Jul 2026)

§8 Summary table — MAS 2.0 column

Currently:  Capital:  ~$30K [tens of $K, est]        Throughput:  high (array)

Change to:  Capital:  ~€150–170K instrument [~€200–300K loaded; sourced: developer]        Throughput:  high (array); up to 0.79–2.07M sequences

IBBIS summary

Summary table — 'MAS 2.0 / AMS (photolithographic)' row, Capital cell

Currently:  ~tens of $K

Change to:  ~€150–170K instrument (~€200–300K loaded; optics ~€150K)

Workstream 1 prose — the MAS 2.0 clause

Currently:  MAS 2.0 (open photolithographic; tens of $K; TRL 5; library-grade output)

Change to:  MAS 2.0 (open photolithographic; ~€150–170K instrument / ~€200–300K end-to-end, optics alone ~€150K; TRL 5; library-grade output; no independent build yet)

Chapter 3 (cost & accessibility) — no change required

Chapter 3 models cost trajectories only for OpenIDS, electrochemical, and the enzymatic benchtop — it does not give MAS 2.0 a cost line, so there is nothing to correct there. OPTIONAL: if you want MAS 2.0 represented, add a one-line note that its capital anchor is now ~€150–170K instrument (~€200–300K loaded), sourced from the developer interview — but it has a single anchor and no trajectory, so no projection is warranted (same TRL-gating logic as electrochemical).

Optional — Chapter 4 scholarly disclosure

When you cite Lietard et al. 2021 (NAR) as the photolithographic error signature, consider a one-line footnote: the lead author, Jory Lietard, is a co-founder of Helices Biological Photolithography, which sells the MAS 2.0. Not a conflict for your use of the published rates — just transparency.